Министерство науки и высшего образования Российской Федерации ФЕДЕРАЛЬНОЕ ГОСУДАРСТВЕННОЕ АВТОНОМНОЕ ОБРАЗОВАТЕЛЬНОЕ УЧРЕЖДЕНИЕ ВЫСШЕГО ОБРАЗОВАНИЯ НАЦИОНАЛЬНЫЙ ИССЛЕДОВАТЕЛЬСКИЙ УНИВЕРСИТЕТ ИТМО (НИУ ИТМО) Факультет программной инженерии и компьютерной техники

ОТЧЁТ О ЛАБОРАТОРНОЙ РАБОТЕ № 2 Администрирование систем и сетей по теме: Создание взаимосвязанной IP-сети Желаемая оценка: 3

Выполнили: студенты группы P3434 Р. Д. Баянов Д. А. Кузнецов Проверил: преподаватель Д. Б. Афанасьев

Санкт-Петербург 2025

СОДЕРЖАНИЕ

План работы	3

1 Ход работы	4

1.1 Настройте основные параметры устройств	4

1.2 Вывод на экран IP-адреса текущего интерфейса и таблицы маршрутизации маршрутизатора	4

1.3 Настройка IP-адреса для физических интерфейсов	4

1.4 Создание loopback-интерфейса	4

1.5 Настройка статического маршрута	4

1.6 Настройка маршрута от R1 к R2 через R3 в качестве резервного маршрута от LoopBacko R1 к LoopBacko R2	4

1.7 Настройка маршрута по умолчанию для установления связи между интерфейсом LoopBacko маршрутизатора R1 и интерфейсом LoopBacko маршрутизатора R2	4

ЗАКЛЮЧЕНИЕ	5

План работы

1. Настройка IP-адресов для интерфейсов на маршрутизаторах.

2. Настройка статических маршрутов для установления связи между

маршрутизаторами.

Ход работы

Настройка основных параметров устройства

router1:

<Huawei>system-view

Enter system view, return user view with Ctrl+Z.

[Huawei]sysname router1

[router1]

router2:

<Huawei>system-view

Enter system view, return user view with Ctrl+Z.

[Huawei]sysname router2

[router2]

router3:

<Huawei>system-view

Enter system view, return user view with Ctrl+Z.

[Huawei]sysname router3

[router3]

Вывод на экран IP-адреса текущего интерфейса и таблицы маршрутизации маршрутизатора

Вывод до конфигурации устройств:

Интерфейсы устройств:

router1:

[router1]display ip interface brief

*down: administratively down

^down: standby

(l): loopback

(s): spoofing

The number of interface that is UP in Physical is 3

The number of interface that is DOWN in Physical is 1

The number of interface that is UP in Protocol is 1

The number of interface that is DOWN in Protocol is 3

Interface                     IP Address/Mask      Physical   Protocol

GigabitEthernet0/0/0              unassigned           up         down

GigabitEthernet0/0/1              unassigned           up         down

GigabitEthernet0/0/2              unassigned           down       down

NULL0                             unassigned           up         up(s)

[router1]

router2:

[router2]display ip interface brief

*down: administratively down

^down: standby

(l): loopback

(s): spoofing

The number of interface that is UP in Physical is 3

The number of interface that is DOWN in Physical is 1

The number of interface that is UP in Protocol is 1

The number of interface that is DOWN in Protocol is 3

Interface                     IP Address/Mask      Physical   Protocol

GigabitEthernet0/0/0              unassigned           up         down

GigabitEthernet0/0/1              unassigned           down       down

GigabitEthernet0/0/2              unassigned           up         down

NULL0                             unassigned           up         up(s)

[router2]

router3:

[router3]display ip interface brief

*down: administratively down

^down: standby

(l): loopback

(s): spoofing

The number of interface that is UP in Physical is 3

The number of interface that is DOWN in Physical is 1

The number of interface that is UP in Protocol is 1

The number of interface that is DOWN in Protocol is 3

Interface                     IP Address/Mask      Physical   Protocol

GigabitEthernet0/0/0              unassigned           down       down

GigabitEthernet0/0/1              unassigned           up         down

GigabitEthernet0/0/2              unassigned           up         down

NULL0                             unassigned           up         up(s)

[router3]

Таблицы маршрутизации:

router1:

[router1]display ip routing-table

Route Flags: R - relay, D - download to fib

------------------------------------------------------------------------------

Routing Tables: Public

Destinations : 4        Routes : 4

Destination/Mask    Proto   Pre  Cost    Flags  NextHop        Interface

127.0.0.0/8   	  Direct  0    0           D   127.0.0.1       InLoopBack0      127.0.0.1/32  	  Direct  0    0           D   127.0.0.1       InLoopBack0

127.255.255.255/32  Direct  0    0          D   127.0.0.1       InLoopBack0

255.255.255.255/32  Direct  0    0           D   127.0.0.1       InLoopBack0

[router1]

router2:

[router2]display ip routing-table

Route Flags: R - relay, D - download to fib

------------------------------------------------------------------------------

Routing Tables: Public

Destinations : 4        Routes : 4

Destination/Mask    Proto   Pre  Cost      Flags NextHop         Interface

127.0.0.0/8         Direct  0    0           D   127.0.0.1       InLoopBack0

127.0.0.1/32  	  Direct  0    0           D   127.0.0.1       InLoopBack0

127.255.255.255/32  Direct  0    0           D   127.0.0.1       InLoopBack0

255.255.255.255/32  Direct  0    0           D   127.0.0.1       InLoopBack0

[router2]

router3:

[router3]display ip routing-table

Route Flags: R - relay, D - download to fib

------------------------------------------------------------------------------

Routing Tables: Public

Destinations : 4        Routes : 4

Destination/Mask    Proto   Pre  Cost      Flags NextHop         Interface

127.0.0.0/8   	  Direct  0    0           D   127.0.0.1       InLoopBack0

127.0.0.1/32        Direct  0    0           D   127.0.0.1       InLoopBack0

127.255.255.255/32  Direct  0    0           D   127.0.0.1       InLoopBack0

255.255.255.255/32  Direct  0    0           D   127.0.0.1       InLoopBack0

[router3]

Настройка IP-адреса для физических интерфейсов

router1:

[router1]interface GigabitEthernet0/0/1

[router1-GigabitEthernet0/0/1]ip address 10.0.13.1 24

Oct  9 2025 05:44:00-08:00 router1 %%01IFNET/4/LINK_STATE(l)[0]:The line protoco

l IP on the interface GigabitEthernet0/0/1 has entered the UP state.

[router1-GigabitEthernet0/0/1]

[router1-GigabitEthernet0/0/1]quit

[router1]

[router1]interface GigabitEthernet0/0/0

[router1-GigabitEthernet0/0/0]ip address 10.0.12.1 24

Oct  9 2025 05:45:22-08:00 router1 %%01IFNET/4/LINK_STATE(l)[1]:The line protoco

l IP on the interface GigabitEthernet0/0/0 has entered the UP state.

[router1-GigabitEthernet0/0/0]quit

[router1]

router2:

[router2]interface GigabitEthernet0/0/0

[router2-GigabitEthernet0/0/0]ip address 10.0.12.2 24

[router2-GigabitEthernet0/0/0]

Oct  9 2025 05:46:37-08:00 router2 %%01IFNET/4/LINK_STATE(l)[0]:The line protoco

l IP on the interface GigabitEthernet0/0/0 has entered the UP state.

[router2-GigabitEthernet0/0/0]quit

[router2]

[router2]interface GigabitEthernet0/0/2

[router2-GigabitEthernet0/0/2]ip address 10.0.23.2 24

[router2-GigabitEthernet0/0/2]

Oct  9 2025 05:47:43-08:00 router2 %%01IFNET/4/LINK_STATE(l)[1]:The line protoco

l IP on the interface GigabitEthernet0/0/2 has entered the UP state.

[router2-GigabitEthernet0/0/2]quit

[router2]

router3:

[router3]interface GigabitEthernet0/0/1

[router3-GigabitEthernet0/0/1]ip address 10.0.13.3 24

Oct  9 2025 05:49:12-08:00 router3 %%01IFNET/4/LINK_STATE(l)[0]:The line protoco

l IP on the interface GigabitEthernet0/0/1 has entered the UP state.

[router3-GigabitEthernet0/0/1]quit

[router3]interface GigabitEthernet0/0/2

[router3-GigabitEthernet0/0/2]ip address 10.0.23.3 24

Oct  9 2025 05:49:49-08:00 router3 %%01IFNET/4/LINK_STATE(l)[1]:The line protoco

l IP on the interface GigabitEthernet0/0/2 has entered the UP state.

[router3-GigabitEthernet0/0/2]quit

[router3]

Проверка наличия связи:

router1:

[router1]ping 10.0.12.2

PING 10.0.12.2: 56  data bytes, press CTRL_C to break

Reply from 10.0.12.2: bytes=56 Sequence=1 ttl=255 time=790 ms

Reply from 10.0.12.2: bytes=56 Sequence=2 ttl=255 time=70 ms

Reply from 10.0.12.2: bytes=56 Sequence=3 ttl=255 time=40 ms

Reply from 10.0.12.2: bytes=56 Sequence=4 ttl=255 time=30 ms

Reply from 10.0.12.2: bytes=56 Sequence=5 ttl=255 time=50 ms

--- 10.0.12.2 ping statistics ---

5 packet(s) transmitted

5 packet(s) received

0.00% packet loss

round-trip min/avg/max = 30/196/790 ms

[router1]

[router1]ping 10.0.13.3

PING 10.0.13.3: 56  data bytes, press CTRL_C to break

Reply from 10.0.13.3: bytes=56 Sequence=1 ttl=255 time=480 ms

Reply from 10.0.13.3: bytes=56 Sequence=2 ttl=255 time=60 ms

Reply from 10.0.13.3: bytes=56 Sequence=3 ttl=255 time=30 ms

Reply from 10.0.13.3: bytes=56 Sequence=4 ttl=255 time=40 ms

Reply from 10.0.13.3: bytes=56 Sequence=5 ttl=255 time=40 ms

--- 10.0.13.3 ping statistics ---

5 packet(s) transmitted

5 packet(s) received

0.00% packet loss

round-trip min/avg/max = 30/130/480 ms

[router1]

router2:

<router2>ping 10.0.12.1

PING 10.0.12.1: 56  data bytes, press CTRL_C to break

Reply from 10.0.12.1: bytes=56 Sequence=1 ttl=255 time=140 ms

Reply from 10.0.12.1: bytes=56 Sequence=2 ttl=255 time=50 ms

Reply from 10.0.12.1: bytes=56 Sequence=3 ttl=255 time=40 ms

Reply from 10.0.12.1: bytes=56 Sequence=4 ttl=255 time=40 ms

Reply from 10.0.12.1: bytes=56 Sequence=5 ttl=255 time=40 ms

--- 10.0.12.1 ping statistics ---

5 packet(s) transmitted

5 packet(s) received

0.00% packet loss

round-trip min/avg/max = 40/62/140 ms

<router2>

<router2>ping 10.0.23.3

PING 10.0.23.3: 56  data bytes, press CTRL_C to break

Reply from 10.0.23.3: bytes=56 Sequence=1 ttl=255 time=260 ms

Reply from 10.0.23.3: bytes=56 Sequence=2 ttl=255 time=50 ms

Reply from 10.0.23.3: bytes=56 Sequence=3 ttl=255 time=50 ms

Reply from 10.0.23.3: bytes=56 Sequence=4 ttl=255 time=40 ms

Reply from 10.0.23.3: bytes=56 Sequence=5 ttl=255 time=60 ms

--- 10.0.23.3 ping statistics ---

5 packet(s) transmitted

5 packet(s) received

0.00% packet loss

round-trip min/avg/max = 40/92/260 ms

<router2>

router3:

<router3>ping 10.0.13.1

PING 10.0.13.1: 56  data bytes, press CTRL_C to break

Reply from 10.0.13.1: bytes=56 Sequence=1 ttl=255 time=140 ms

Reply from 10.0.13.1: bytes=56 Sequence=2 ttl=255 time=50 ms

Reply from 10.0.13.1: bytes=56 Sequence=3 ttl=255 time=40 ms

Reply from 10.0.13.1: bytes=56 Sequence=4 ttl=255 time=20 ms

Reply from 10.0.13.1: bytes=56 Sequence=5 ttl=255 time=40 ms

--- 10.0.13.1 ping statistics ---

5 packet(s) transmitted

5 packet(s) received

0.00% packet loss

round-trip min/avg/max = 20/58/140 ms

<router3>

<router3>ping 10.0.23.2

PING 10.0.23.2: 56  data bytes, press CTRL_C to break

Reply from 10.0.23.2: bytes=56 Sequence=1 ttl=255 time=50 ms

Reply from 10.0.23.2: bytes=56 Sequence=2 ttl=255 time=50 ms

Reply from 10.0.23.2: bytes=56 Sequence=3 ttl=255 time=40 ms

Reply from 10.0.23.2: bytes=56 Sequence=4 ttl=255 time=40 ms

Reply from 10.0.23.2: bytes=56 Sequence=5 ttl=255 time=40 ms

--- 10.0.23.2 ping statistics ---

5 packet(s) transmitted

5 packet(s) received

0.00% packet loss

round-trip min/avg/max = 40/44/50 ms

<router3>

Таблицы маршрутизации после конфигурации:

router1:

<router1>display ip routing-table

Route Flags: R - relay, D - download to fib

------------------------------------------------------------------------------

Routing Tables: Public

Destinations : 10       Routes : 10

Destination/Mask    Proto   Pre  Cost      Flags NextHop         Interface

10.0.12.0/24  Direct  0    0           D   10.0.12.1       GigabitEthernet

0/0/0

10.0.12.1/32  Direct  0    0           D   127.0.0.1       GigabitEthernet

0/0/0

10.0.12.255/32  Direct  0    0           D   127.0.0.1       GigabitEthernet

0/0/0

10.0.13.0/24  Direct  0    0           D   10.0.13.1       GigabitEthernet

0/0/1

10.0.13.1/32  Direct  0    0           D   127.0.0.1       GigabitEthernet

0/0/1

10.0.13.255/32  Direct  0    0           D   127.0.0.1       GigabitEthernet

0/0/1

127.0.0.0/8   Direct  0    0           D   127.0.0.1       InLoopBack0

127.0.0.1/32  Direct  0    0           D   127.0.0.1       InLoopBack0

127.255.255.255/32  Direct  0    0           D   127.0.0.1       InLoopBack0

255.255.255.255/32  Direct  0    0           D   127.0.0.1       InLoopBack0

<router1>

router2:

<router2>display ip routing-table

Route Flags: R - relay, D - download to fib

------------------------------------------------------------------------------

Routing Tables: Public

Destinations : 10       Routes : 10

Destination/Mask    Proto   Pre  Cost      Flags NextHop         Interface

10.0.12.0/24  Direct  0    0           D   10.0.12.2       GigabitEthernet

0/0/0

10.0.12.2/32  Direct  0    0           D   127.0.0.1       GigabitEthernet

0/0/0

10.0.12.255/32  Direct  0    0           D   127.0.0.1       GigabitEthernet

0/0/0

10.0.23.0/24  Direct  0    0           D   10.0.23.2       GigabitEthernet

0/0/2

10.0.23.2/32  Direct  0    0           D   127.0.0.1       GigabitEthernet

0/0/2

10.0.23.255/32  Direct  0    0           D   127.0.0.1       GigabitEthernet

0/0/2

127.0.0.0/8   Direct  0    0           D   127.0.0.1       InLoopBack0

127.0.0.1/32  Direct  0    0           D   127.0.0.1       InLoopBack0

127.255.255.255/32  Direct  0    0           D   127.0.0.1       InLoopBack0

255.255.255.255/32  Direct  0    0           D   127.0.0.1       InLoopBack0

router3:

<router3>display ip routing-table

Route Flags: R - relay, D - download to fib

------------------------------------------------------------------------------

Routing Tables: Public

Destinations : 10       Routes : 10

Destination/Mask    Proto   Pre  Cost      Flags NextHop         Interface

10.0.13.0/24  Direct  0    0           D   10.0.13.3       GigabitEthernet

0/0/1

10.0.13.3/32  Direct  0    0           D   127.0.0.1       GigabitEthernet

0/0/1

10.0.13.255/32  Direct  0    0           D   127.0.0.1       GigabitEthernet

0/0/1

10.0.23.0/24  Direct  0    0           D   10.0.23.3       GigabitEthernet

0/0/2

10.0.23.3/32  Direct  0    0           D   127.0.0.1       GigabitEthernet

0/0/2

10.0.23.255/32  Direct  0    0           D   127.0.0.1       GigabitEthernet

0/0/2

127.0.0.0/8   Direct  0    0           D   127.0.0.1       InLoopBack0

127.0.0.1/32  Direct  0    0           D   127.0.0.1       InLoopBack0

127.255.255.255/32  Direct  0    0           D   127.0.0.1       InLoopBack0

255.255.255.255/32  Direct  0    0           D   127.0.0.1       InLoopBack0

<router3>

Создание loopback-интерфейса

Настройка статического маршрута

Настройка маршрута от R1 к R2 через R3 в качестве резервного маршрута от LoopBacko R1 к LoopBacko R2

Настройка маршрута по умолчанию для установления связи между интерфейсом LoopBacko маршрутизатора R1 и интерфейсом LoopBacko маршрутизатора R2

ЗАКЛЮЧЕНИЕ

Цель работы достигнута.