Министерство науки и высшего образования Российской Федерации ФЕДЕРАЛЬНОЕ ГОСУДАРСТВЕННОЕ АВТОНОМНОЕ ОБРАЗОВАТЕЛЬНОЕ УЧРЕЖДЕНИЕ ВЫСШЕГО ОБРАЗОВАНИЯ НАЦИОНАЛЬНЫЙ ИССЛЕДОВАТЕЛЬСКИЙ УНИВЕРСИТЕТ ИТМО (НИУ ИТМО) Факультет программной инженерии и компьютерной техники

ОТЧЁТ О ЛАБОРАТОРНОЙ РАБОТЕ № 3 Администрирование систем и сетей по теме: Создание коммутируемой сети Ethernet Часть 1: Основы Ethernet и конфигурирование VLAN Желаемая оценка: 3

Выполнили: студенты группы P3434 Р. Д. Баянов Д. А. Кузнецов Проверил: преподаватель А. Н. Максимов

Санкт-Петербург 2025

СОДЕРЖАНИЕ

План работы	3

1 Ход работы	4

1.1 Настройка основных параметров устройства	4

1.2 Настройка IP-адресов	5

1.3 Создание VLAN	7

1.4 Настройка сети VLAN на основе портов	7

1.5 Конфигурирование сети VLAN на основе MAC-адресов	8

1.6 Вывод информации о конфигурации VLAN	8

ЗАКЛЮЧЕНИЕ	13

План работы

1. Создание VLAN.

2. Конфигурирование VLAN на основе портов.

3. Конфигурирование VLAN на основе MAC-адресов.

Ход работы

Настройка основных параметров устройства

S1:

<Huawei>system-view

Enter system view, return user view with Ctrl+Z.

[Huawei]sysname S1

[S1]

S2:

<Huawei>system-view

Enter system view, return user view with Ctrl+Z.

[Huawei]sysname S2

[S2]

S3:

<Huawei>system-view

Enter system view, return user view with Ctrl+Z.

[Huawei]sysname S3

[S3]

S4:

<Huawei>system-view

Enter system view, return user view with Ctrl+Z.

[Huawei]sysname S4

[S4]

R1:

<Huawei>system-view

Enter system view, return user view with Ctrl+Z.

[Huawei]sysname R1

R4:

<Huawei>system-view

Enter system view, return user view with Ctrl+Z.

[Huawei]sysname R4

[R4]

Отключение ненужных портов S1:

[S1]interface GigabitEthernet 0/0/11

[S1-GigabitEthernet0/0/11]shutdown

[S1-GigabitEthernet0/0/11]quit

[S1]

Oct 18 2025 11:29:13-08:00 S1 DS/4/DATASYNC_CFGCHANGE:OID 1.3.6.1.4.1.2011.5.25.

191.3.1 configurations have been changed. The current change number is 5, the ch

ange loop count is 0, and the maximum number of records is 4095.

[S1]interface GigabitEthernet 0/0/12

[S1-GigabitEthernet0/0/12]shutdown

[S1-GigabitEthernet0/0/12]quit

[S1]

Oct 18 2025 11:29:33-08:00 S1 DS/4/DATASYNC_CFGCHANGE:OID 1.3.6.1.4.1.2011.5.25.

191.3.1 configurations have been changed. The current change number is 6, the ch

ange loop count is 0, and the maximum number of records is 4095.

Отключение ненужных портов S2:

[S2]interface GigabitEthernet 0/0/11

[S2-GigabitEthernet0/0/11]shutdown

[S2-GigabitEthernet0/0/11]quit

[S2]interface GigabitEthernet 0/0/12

[S2-GigabitEthernet0/0/12]shutdown

[S2-GigabitEthernet0/0/12]quit

Oct 18 2025 11:30:36-08:00 S2 DS/4/DATASYNC_CFGCHANGE:OID 1.3.6.1.4.1.2011.5.25.

191.3.1 configurations have been changed. The current change number is 6, the ch

ange loop count is 0, and the maximum number of records is 4095.

[S2]

Настройка IP-адресов

Настройка IP на R1 и R3:

[R1]interface GigabitEthernet 0/0/1

[R1-GigabitEthernet0/0/1]ip address 10.1.2.1 24

[R1-GigabitEthernet0/0/1]

Oct 19 2025 02:32:05-08:00 R1 %%01IFNET/4/LINK_STATE(l)[0]:The line protocol IP

on the interface GigabitEthernet0/0/1 has entered the UP state.

[R3]interface GigabitEthernet 0/0/2

[R3-GigabitEthernet0/0/2]ip address 10.1.10.1 24

[R3-GigabitEthernet0/0/2]

Oct 19 2025 02:33:08-08:00 R3 %%01IFNET/4/LINK_STATE(l)[0]:The line protocol IP

on the interface GigabitEthernet0/0/2 has entered the UP state.

[R3-GigabitEthernet0/0/2]

Коммутатор не поддерживает L3

[S3-GigabitEthernet0/0/1]interface gigabitethernet 0/0/1

[S3-GigabitEthernet0/0/1]undo portswitch

Error: The interface is already a L3 interface.

[S3-GigabitEthernet0/0/1]ip address 10.1.3.1 24

^

Error: Unrecognized command found at '^' position.

Создание VLAN на S3 и S4:

[S3]vlan 3

[S3-vlan3]

[S4]vlan 3

[S4-vlan3]

Настройка портов на S3 и S4 в качестве портов доступа:

[S3]interface GigabitEthernet 0/0/1

[S3-GigabitEthernet0/0/1]port link-type access

[S3-GigabitEthernet0/0/1]port default vlan 3

[S3-GigabitEthernet0/0/1]quit

[S4]interface GigabitEthernet 0/0/2

[S4-GigabitEthernet0/0/2]port link-type access

[S4-GigabitEthernet0/0/2]port default vlan 3

[S4-GigabitEthernet0/0/2]quit

Создание VLANIF интерфейса и настройка IP адресов:

[S3] interface Vlanif3

[S3-Vlanif3]ip address 10.1.3.1 24

[S4]interface Vlanif3

[S4-Vlanif3]ip address 10.1.3.2 24

Создание VLAN

[S1]vlan batch 2 to 3 10

info: This operation may take a few seconds. Please wait for a moment...done.

[S2]vlan batch 2 to 3 10

info: This operation may take a few seconds. Please wait for a moment...done.

Настройка сети VLAN на основе портов

Настройка VLAN интерфейсов на устройствах:

[S1]interface GigabitEthernet 0/0/1

[S1-GigabitEthernet0/0/1]port link-type access

[S1-GigabitEthernet0/0/1]port default vlan 2

[S1-GigabitEthernet0/0/1]quit

[S1]interface GigabitEthernet 0/0/13

[S1-GigabitEthernet0/0/13]port link-type access

[S1-GigabitEthernet0/0/13]port default vlan 3 [S1-GigabitEthernet0/0/13]quit

[S2]interface GigabitEthernet 0/0/14

[S2-GigabitEthernet0/0/14]port link-type access

[S2-GigabitEthernet0/0/14]port default vlan 3 [S2-GigabitEthernet0/0/14]quit

Настройка магистральных портов:

[S1]interface GigabitEthernet 0/0/10

[S1-GigabitEthernet0/0/10]port link-type trunk

[S1-GigabitEthernet0/0/10]port trunk allow-pass vlan 2 3 [S1-GigabitEthernet0/0/10]undo port trunk allow-pass vlan 1

[S2]interface GigabitEthernet 0/0/10

[S2-GigabitEthernet0/0/10]port link-type trunk

[S2-GigabitEthernet0/0/10]port trunk allow-pass vlan 2 3 [S2-GigabitEthernet0/0/10]undo port trunk allow-pass vlan 1

Конфигурирование сети VLAN на основе MAC-адресов

Устанавливаем на S2 маппинг MAC адреса на VLAN 10

[S2] vlan 10

[S2-vlan10] mac-vlan mac-address a008-6fe1-0c46

Меняем тип портов на гибридный и указываем что трафик с VLAN 10 будет передаваться без тега:

[S2]interface GigabitEthernet 0/0/1

[S2-GigabitEthernet0/0/1]port link-type hybrid

[S2-GigabitEthernet0/0/1]port hybrid untagged vlan 10

[S2-GigabitEthernet0/0/1]quit

[S2]interface GigabitEthernet 0/0/2

[S2-GigabitEthernet0/0/2]port link-type hybrid

[S2-GigabitEthernet0/0/2]port hybrid untagged vlan 10

[S2-GigabitEthernet0/0/2]quit

[S2]interface GigabitEthernet 0/0/3

[S2-GigabitEthernet0/0/3]port link-type hybrid

[S2-GigabitEthernet0/0/3]port hybrid untagged vlan 10

[S2-GigabitEthernet0/0/3]quit

Разрешаем передачу VLAN 10 на S1 и S2:

[S1]interface GigabitEthernet0/0/10

[S1-GigabitEthernet0/0/10]port trunk allow-pass vlan 10 [S1-GigabitEthernet0/0/10]quit

[S2]interface GigabitEthernet0/0/10

[S2-GigabitEthernet0/0/10]port trunk allow-pass vlan 10 [S2-GigabitEthernet0/0/10]quit

Включаем на всех портах S2, которые связаны с R3 назначенные VLAN на основе MAC:

[S2]interface GigabitEthernet0/0/1

[S2-GigabitEthernet0/0/1]mac-vlan enable [S2-GigabitEthernet0/0/1]quit

[S2]interface GigabitEthernet0/0/2

[S2-GigabitEthernet0/0/2]mac-vlan enable

[S2-GigabitEthernet0/0/2]quit

[S2]interface GigabitEthernet0/0/3

[S2-GigabitEthernet0/0/3]mac-vlan enable

[S2-GigabitEthernet0/0/3]quit

Вывод информации о конфигурации VLAN

[S1]display vlan

The total number of vlans is : 4

--------------------------------------------------------------------------------

U: Up;         D: Down;         TG: Tagged;         UT: Untagged;

MP: Vlan-mapping;               ST: Vlan-stacking;

#: ProtocolTransparent-vlan;    *: Management-vlan;

--------------------------------------------------------------------------------

VID  Type    Ports

--------------------------------------------------------------------------------

1    common  UT:GE0/0/2(D)      GE0/0/3(D)      GE0/0/4(D)      GE0/0/5(D)

GE0/0/6(D)      GE0/0/7(D)      GE0/0/8(D)      GE0/0/9(D)

GE0/0/11(D)     GE0/0/12(D)     GE0/0/14(D)     GE0/0/15(D)

GE0/0/16(D)     GE0/0/17(D)     GE0/0/18(D)     GE0/0/19(D)

GE0/0/20(D)     GE0/0/21(D)     GE0/0/22(D)     GE0/0/23(D)

GE0/0/24(D)

2    common  UT:GE0/0/1(U)

TG:GE0/0/10(U)

3    common  UT:GE0/0/13(U)

TG:GE0/0/10(U)

10   common  TG:GE0/0/10(U)

VID  Status  Property      MAC-LRN Statistics Description

--------------------------------------------------------------------------------

1    enable  default       enable  disable    VLAN 0001

2    enable  default       enable  disable    VLAN 0002

3    enable  default       enable  disable    VLAN 0003

10   enable  default       enable  disable    VLAN 0010

[S1]

[S2]display vlan

The total number of vlans is : 4

--------------------------------------------------------------------------------

U: Up;         D: Down;         TG: Tagged;         UT: Untagged;

MP: Vlan-mapping;               ST: Vlan-stacking;

#: ProtocolTransparent-vlan;    *: Management-vlan;

--------------------------------------------------------------------------------

VID  Type    Ports

--------------------------------------------------------------------------------

1    common  UT:GE0/0/1(U)      GE0/0/2(D)      GE0/0/3(D)      GE0/0/4(D)

GE0/0/5(D)      GE0/0/6(D)      GE0/0/7(D)      GE0/0/8(D)

GE0/0/9(D)      GE0/0/11(D)     GE0/0/12(D)     GE0/0/13(D)

GE0/0/15(D)     GE0/0/16(D)     GE0/0/17(D)     GE0/0/18(D)

GE0/0/19(D)     GE0/0/20(D)     GE0/0/21(D)     GE0/0/22(D)

GE0/0/23(D)     GE0/0/24(D)

2    common  TG:GE0/0/10(U)

3    common  UT:GE0/0/14(U)

TG:GE0/0/10(U)

10   common  UT:GE0/0/1(U)      GE0/0/2(D)      GE0/0/3(D)

TG:GE0/0/10(U)

VID  Status  Property      MAC-LRN Statistics Description

--------------------------------------------------------------------------------

1    enable  default       enable  disable    VLAN 0001

2    enable  default       enable  disable    VLAN 0002

3    enable  default       enable  disable    VLAN 0003

10   enable  default       enable  disable    VLAN 0010

[S2]

Конфигурация MAC-VLAN на S2:

[S2]display mac-vlan vlan 10

---------------------------------------------------

MAC Address     MASK            VLAN    Priority

---------------------------------------------------

a008-6fe1-0c46  ffff-ffff-ffff  10      0

Total MAC VLAN address count: 1

[S2]

Проверка конфигурации:

<S4>ping 10.1.3.1

PING 10.1.3.1: 56  data bytes, press CTRL_C to break

Reply from 10.1.3.1: bytes=56 Sequence=1 ttl=255 time=190 ms

Reply from 10.1.3.1: bytes=56 Sequence=2 ttl=255 time=100 ms

Reply from 10.1.3.1: bytes=56 Sequence=3 ttl=255 time=90 ms

Reply from 10.1.3.1: bytes=56 Sequence=4 ttl=255 time=100 ms

Reply from 10.1.3.1: bytes=56 Sequence=5 ttl=255 time=110 ms

--- 10.1.3.1 ping statistics ---

5 packet(s) transmitted

5 packet(s) received

0.00% packet loss

round-trip min/avg/max = 90/118/190 ms

<S4>

Отсутствие response при ping с R1:

<R1>ping 10.1.3.1

PING 10.1.3.1: 56  data bytes, press CTRL_C to break

Request time out

Request time out

Request time out

Request time out

Request time out

--- 10.1.3.1 ping statistics ---

5 packet(s) transmitted

0 packet(s) received

100.00% packet loss

<R1>

<R1>ping 10.1.10.1

PING 10.1.10.1: 56  data bytes, press CTRL_C to break

Request time out

Request time out

Request time out

Request time out

Request time out

--- 10.1.10.1 ping statistics ---

5 packet(s) transmitted

0 packet(s) received

100.00% packet loss

<R1>

<R1>ping 10.1.3.2

PING 10.1.3.2: 56  data bytes, press CTRL_C to break

Request time out

Request time out

Request time out

Request time out

Request time out

--- 10.1.3.2 ping statistics ---

5 packet(s) transmitted

0 packet(s) received

100.00% packet loss

<R1>

Таблицы MAC адресов на коммутаторах:

[S1]display mac-address verbose

MAC address table of slot 0:

-------------------------------------------------------------------------------

MAC Address    VLAN/       PEVLAN CEVLAN Port            Type      LSP/LSR-ID

VSI/SI                                              MAC-Tunnel

-------------------------------------------------------------------------------

4c1f-cc95-463e 3           -      -      GE0/0/10        dynamic   0/-

4c1f-cc9d-3d89 3           -      -      GE0/0/13        dynamic   0/-

-------------------------------------------------------------------------------

Total matching items on slot 0 displayed = 2

[S1]

<S2>display mac-address verbose

MAC address table of slot 0:

-------------------------------------------------------------------------------

MAC Address    VLAN/       PEVLAN CEVLAN Port            Type      LSP/LSR-ID

VSI/SI                                              MAC-Tunnel

-------------------------------------------------------------------------------

4c1f-cc95-463e 3           -      -      GE0/0/14        dynamic   0/-

4c1f-cc9d-3d89 3           -      -      GE0/0/10        dynamic   0/-

-------------------------------------------------------------------------------

Total matching items on slot 0 displayed = 2

<S2>

ЗАКЛЮЧЕНИЕ

Цель работы достигнута.