Министерство науки и высшего образования Российской Федерации ФЕДЕРАЛЬНОЕ ГОСУДАРСТВЕННОЕ АВТОНОМНОЕ ОБРАЗОВАТЕЛЬНОЕ УЧРЕЖДЕНИЕ ВЫСШЕГО ОБРАЗОВАНИЯ НАЦИОНАЛЬНЫЙ ИССЛЕДОВАТЕЛЬСКИЙ УНИВЕРСИТЕТ ИТМО (НИУ ИТМО) Факультет программной инженерии и компьютерной техники

ОТЧЁТ О ЛАБОРАТОРНОЙ РАБОТЕ № 3 Администрирование систем и сетей по теме: Основы сетевой безопасности и доступа к сети Часть 3: Настройка NAT Желаемая оценка: 3

Выполнили: студенты группы P3434 Р. Д. Баянов Д. А. Кузнецов Проверил: преподаватель А. Н. Максимов

Санкт-Петербург 2025

СОДЕРЖАНИЕ

План работы	3

1 Ход работы	4

1.1 Настройка основных параметров устройства	4

1.2 Настройка динамического NAT	6

1.3 Настройка сервера NAT на исходящем интерфейсе R2	7

ЗАКЛЮЧЕНИЕ	9

План работы

1. Настройка динамического NAT.

2. Настройка Easy IP.

3. Настройка сервера NAT.

Ход работы

Настройка основных параметров устройства

<Huawei>system-view

Enter system view, return user view with Ctrl+Z.

[Huawei]sysname R1

[R1]

<Huawei>system-view

Enter system view, return user view with Ctrl+Z.

[Huawei]sysname R2

<Huawei>system-view

Enter system view, return user view with Ctrl+Z.

[Huawei]sysname R3

[R3]

Настройка ip адресов и маршрутов по умолчанию

[R1] interface GigabitEthernet 0/0/0

[R1-GigabitEthernet0/0/0] ip address 192.168.1.1 24

[R1-GigabitEthernet0/0/0] quit

[R1]ip route-static 0.0.0.0 0 192.168.1.254

[R2] interface GigabitEthernet 0/0/0

[R2-GigabitEthernet0/0/0] ip address 192.168.1.254 24

[R2-GigabitEthernet0/0/0] quit

[R2] interface GigabitEthernet 0/0/1

[R2-GigabitEthernet0/0/1] ip address 1.2.3.4 24

[R2-GigabitEthernet0/0/1] quit

[R2] ip route-static 0.0.0.0 0 1.2.3.254

[R3] interface GigabitEthernet 0/0/0

[R3-GigabitEthernet0/0/0] ip address 1.2.3.254 24

Настройка Telnet

[R1]user-interface vty 0 4

[R1-ui-vty0-4]authentication-mode aaa

[R1-ui-vty0-4]quit

[R1]aaa

[R1-aaa]local-user test password cipher test

Info: Add a new user.

[R1-aaa]local-user test service-type telnet

[R1-aaa]local-user test privilege level 15

[R3-GigabitEthernet0/0/0]user-interface vty 0 4

[R3-ui-vty0-4]authentication-mode aaa

[R3-ui-vty0-4]quit

[R3]aaa

[R3-aaa]local-user test password cipher test

Info: Add a new user.

[R3-aaa]local-user test service-type telnet

[R3-aaa]local-user test privilege level 15

[R3-aaa]quit

[R3]

Проверка возможности установления связи

[R1]ping 1.2.3.254

PING 1.2.3.254: 56  data bytes, press CTRL_C to break

Request time out

Request time out

Request time out

Request time out

Request time out

--- 1.2.3.254 ping statistics ---

5 packet(s) transmitted

0 packet(s) received

100.00% packet loss

[R2]ping 1.2.3.254

PING 1.2.3.254: 56  data bytes, press CTRL_C to break

Reply from 1.2.3.254: bytes=56 Sequence=1 ttl=255 time=400 ms

Reply from 1.2.3.254: bytes=56 Sequence=2 ttl=255 time=70 ms

Reply from 1.2.3.254: bytes=56 Sequence=3 ttl=255 time=40 ms

Reply from 1.2.3.254: bytes=56 Sequence=4 ttl=255 time=40 ms

Reply from 1.2.3.254: bytes=56 Sequence=5 ttl=255 time=50 ms

--- 1.2.3.254 ping statistics ---

5 packet(s) transmitted

5 packet(s) received

0.00% packet loss

round-trip min/avg/max = 40/120/400 ms

[R2]

Настройка динамического NAT

Настройка пула NAT

[R2] nat address-group 1 1.2.3.10 1.2.3.20

Настройка ACL

[R2]acl 2000

[R2-acl-basic-2000]rule 5 permit source any

Настройка динамического NAT на GigabitEthernet 0/0/1 маршрутизатора R2

[R2-acl-basic-2000]interface GigabitEthernet 0/0/1

[R2-GigabitEthernet0/0/1]nat outbound 2000 address-group 1

Проверка связи между R1 и R3

<R1>ping 1.2.3.254

PING 1.2.3.254: 56  data bytes, press CTRL_C to break

Reply from 1.2.3.254: bytes=56 Sequence=1 ttl=254 time=400 ms

Reply from 1.2.3.254: bytes=56 Sequence=2 ttl=254 time=50 ms

Reply from 1.2.3.254: bytes=56 Sequence=3 ttl=254 time=70 ms

Reply from 1.2.3.254: bytes=56 Sequence=4 ttl=254 time=60 ms

Reply from 1.2.3.254: bytes=56 Sequence=5 ttl=254 time=30 ms

--- 1.2.3.254 ping statistics ---

5 packet(s) transmitted

5 packet(s) received

0.00% packet loss

round-trip min/avg/max = 30/122/400 ms

Проверка входа на R3 при помощи Telnet

<R1>telnet 1.2.3.254

Press CTRL_] to quit telnet mode

Trying 1.2.3.254 ...

Connected to 1.2.3.254 ...

Login authentication

Username:test

Password:

<R3>

Таблица NAT сеансов на R2

<R2>display nat session all

NAT Session Table Information:

Protocol          : TCP(6)

SrcAddr  Port Vpn : 192.168.1.1     18114

DestAddr Port Vpn : 1.2.3.254       5888

NAT-Info

New SrcAddr     : 1.2.3.16

New SrcPort     : 10241

New DestAddr    : ----

New DestPort    : ----

Total : 1

Настройка сервера NAT на исходящем интерфейсе R2

[R2]interface GigabitEthernet 0/0/1

[R2-GigabitEthernet0/0/1]nat server protocol tcp global current-interface 2323 inside 192.168.1.1 telnet

Проверка установления соединения между R3 и R1

<R3>telnet 1.2.3.4 2323

Press CTRL_] to quit telnet mode

Trying 1.2.3.4 ...

Connected to 1.2.3.4 ...

Login authentication

Username:test

Password:

<R1>

Таблица сеансов на R2

<R2>display nat session all

NAT Session Table Information:

Protocol          : TCP(6)

SrcAddr  Port Vpn : 1.2.3.254       14530

DestAddr Port Vpn : 1.2.3.4         4873

NAT-Info

New SrcAddr     : ----

New SrcPort     : ----

New DestAddr    : 192.168.1.1

New DestPort    : 5888

Protocol          : TCP(6)

SrcAddr  Port Vpn : 192.168.1.1     28870

DestAddr Port Vpn : 1.2.3.254       5888

NAT-Info

New SrcAddr     : 1.2.3.16

New SrcPort     : 10241

New DestAddr    : ----

New DestPort    : ----

Total : 2

ЗАКЛЮЧЕНИЕ

Цель работы достигнута.