Министерство науки и высшего образования Российской Федерации ФЕДЕРАЛЬНОЕ ГОСУДАРСТВЕННОЕ АВТОНОМНОЕ ОБРАЗОВАТЕЛЬНОЕ УЧРЕЖДЕНИЕ ВЫСШЕГО ОБРАЗОВАНИЯ НАЦИОНАЛЬНЫЙ ИССЛЕДОВАТЕЛЬСКИЙ УНИВЕРСИТЕТ ИТМО (НИУ ИТМО) Факультет программной инженерии и компьютерной техники

ОТЧЁТ О ЛАБОРАТОРНОЙ РАБОТЕ № 3 Проектирование вычислительных систем по теме: Таймеры Вариант 3

Выполнили: студенты группы P3434 Р. Д. Баянов Д. А. Кузнецов Проверил: преподаватель В. Ю. Пинкевич

Санкт-Петербург 2025

СОДЕРЖАНИЕ

Цель работы	3

Задание	4

Вариант	5

1 Описание организации программы и структуры драйверов, блок-схема прикладного алгоритма	6

2 Исходный код	10

ЗАКЛЮЧЕНИЕ	21

Цель работы

1. Получить базовые знания об устройстве и режимах работы таймеров в микроконтроллерах.

2. Получить навыки использования таймеров и прерываний от таймеров.

3. Получить навыки использования аппаратных каналов ввода-вывода таймеров.

Задание

Разработать программу, которая использует таймеры для управления яркостью светодиодов и излучателем звука (по прерыванию или с использованием аппаратных каналов). Блокирующее ожидание (функция HAL_Delay()) в программе использоваться не должно. Стенд должен поддерживать связь с компьютером по UART и выполнять указанные действия в качестве реакции на нажатие кнопок на клавиатуре компьютера. В данной лабораторной работе каждая нажатая кнопка (символ, отправленный с компьютера на стенд) обрабатываются отдельно, ожидание ввода полной строки не требуется. Для работы с UART на стенде можно использован один из двух вариантов драйвера (по прерыванию и по опросу) на выбор исполнителя. Поддержка двух вариантов не требуется. Частота синхросигнала процессорного ядра и сигнала ШИМ для управления яркостью светодиодов (если используется) должны соответствовать указанным в варианте задания.

Вариант

Реализовать имитатор гирлянды с плавными включениями/выключениями светодиодов и переходами между цветами. Должно быть предусмотрено четыре разных предустановленных режима анимации и один пользовательский режим, который можно настраивать. В каждом режиме должно быть задействовано минимум два светодиода, при этом не обязательно, чтобы они могли гореть одновременно. Действия стенда при получении символов от компьютера:

| Символ | Действие |
| --- | --- |
| «1» | Переход на следующий режим. |
| «2» | Возврат к предыдущему режиму. |
| «3» | Ускорение воспроизведения анимации на 10 % от текущей скорости. |
| «4» | Замедление воспроизведения анимации на 10 % от текущей скорости. |
| «5» | Вывод в UART параметров текущего сохраненного пользовательского режима. |
| «Enter» | Вход в меню настройки. |
| На усмотрение исполнителей | Ввод параметров пользовательского режима: цвет светодиода, яркость, необходимость плавного перехода, длительность, конец последовательности и т. п. |

После ввода каждого символа в UART должно выводиться сообщение о том, какой режим активирован, или текущие настройки, вводимые в меню. Частота процессорного ядра – 90 МГц, частота ШИМ-сигнала для светодиодов – 4000 Гц

Описание организации программы и структуры драйверов, блок-схема прикладного алгоритма

Конфигурация микроконтроллера в STM32

Конфигурация таймера

Блок-схема

Исходный код

/* USER CODE BEGIN Header */ /** ****************************************************************************** * @file : main.c * @brief : Main program body ****************************************************************************** * @attention * * Copyright (c) 2025 STMicroelectronics. * All rights reserved. * * This software is licensed under terms that can be found in the LICENSE file * in the root directory of this software component. * If no LICENSE file comes with this software, it is provided AS-IS. * ****************************************************************************** */ /* USER CODE END Header */ /* Includes ------------------------------------------------------------------*/ #include "main.h" #include "tim.h" #include "usart.h" #include "gpio.h" /* Private includes ----------------------------------------------------------*/ /* USER CODE BEGIN Includes */ /* USER CODE END Includes */ /* Private typedef -----------------------------------------------------------*/ /* USER CODE BEGIN PTD */ typedef enum { BREATH = 0, STROBE, SIREN, EVERY_SIGNAL, CUSTOM_MODE } mode; typedef enum { GREEN, YELLOW, RED } indicator_color; typedef struct { indicator_color color; uint32_t brightness; uint32_t duration; } custom_mode_step; typedef enum { MAIN, CUSTOM } menu_state; /* USER CODE END PTD */ /* Private define ------------------------------------------------------------*/ /* USER CODE BEGIN PD */ #define MIN_BRIGHTNESS 0 #define MAX_BRIGHTNESS 100 #define MAX_CUSTOM_MODE_STEP_COUNT 5 /* USER CODE END PD */ /* Private macro -------------------------------------------------------------*/ /* USER CODE BEGIN PM */ /* USER CODE END PM */ /* Private variables ---------------------------------------------------------*/ /* USER CODE BEGIN PV */ uint32_t speed_percents = 50; mode current_mode = BREATH; char* mode_name[] = { "breath", "strobe", "siren", "every signal", "custom mode" }; custom_mode_step custom_mode_steps[MAX_CUSTOM_MODE_STEP_COUNT]; uint32_t current_custom_mode_step_count = 0; uint32_t current_custom_mode_step_index = 0; uint8_t input_buffer; uint8_t output_buffer[256]; menu_state current_menu_state = MAIN; /* USER CODE END PV */ /* Private function prototypes -----------------------------------------------*/ void SystemClock_Config(void); /* USER CODE BEGIN PFP */ void wait(uint32_t delay); void play_breath(); void play_strobe(); void play_siren(); void play_every_signal(); void play_custom_mode(); /* USER CODE END PFP */ /* Private user code ---------------------------------------------------------*/ /* USER CODE BEGIN 0 */ /* USER CODE END 0 */ /** * @brief The application entry point. * @retval int */ int main(void) { /* USER CODE BEGIN 1 */ /* USER CODE END 1 */ /* MCU Configuration--------------------------------------------------------*/ /* Reset of all peripherals, Initializes the Flash interface and the Systick. */ HAL_Init(); /* USER CODE BEGIN Init */ /* USER CODE END Init */ /* Configure the system clock */ SystemClock_Config(); /* USER CODE BEGIN SysInit */ /* USER CODE END SysInit */ /* Initialize all configured peripherals */ MX_GPIO_Init(); MX_TIM4_Init(); MX_USART6_UART_Init(); /* USER CODE BEGIN 2 */ /* USER CODE END 2 */ /* Infinite loop */ /* USER CODE BEGIN WHILE */ HAL_TIM_PWM_Start(&htim4, TIM_CHANNEL_2); HAL_TIM_PWM_Start(&htim4, TIM_CHANNEL_3); HAL_TIM_PWM_Start(&htim4, TIM_CHANNEL_4); HAL_UART_Receive_IT(&huart6, &input_buffer, 1); while (1) { if (current_mode == BREATH) { play_breath(); } else if (current_mode == STROBE) { play_strobe(); } else if (current_mode == SIREN) { play_siren(); } else if (current_mode == EVERY_SIGNAL) { play_every_signal(); } else { play_custom_mode(); } /* USER CODE END WHILE */ /* USER CODE BEGIN 3 */ } /* USER CODE END 3 */ } /** * @brief System Clock Configuration * @retval None */ void SystemClock_Config(void) { RCC_OscInitTypeDef RCC_OscInitStruct = {0}; RCC_ClkInitTypeDef RCC_ClkInitStruct = {0}; /** Configure the main internal regulator output voltage */ __HAL_RCC_PWR_CLK_ENABLE(); __HAL_PWR_VOLTAGESCALING_CONFIG(PWR_REGULATOR_VOLTAGE_SCALE3); /** Initializes the RCC Oscillators according to the specified parameters * in the RCC_OscInitTypeDef structure. */ RCC_OscInitStruct.OscillatorType = RCC_OSCILLATORTYPE_HSE; RCC_OscInitStruct.HSEState = RCC_HSE_ON; RCC_OscInitStruct.PLL.PLLState = RCC_PLL_ON; RCC_OscInitStruct.PLL.PLLSource = RCC_PLLSOURCE_HSE; RCC_OscInitStruct.PLL.PLLM = 15; RCC_OscInitStruct.PLL.PLLN = 108; RCC_OscInitStruct.PLL.PLLP = RCC_PLLP_DIV2; RCC_OscInitStruct.PLL.PLLQ = 4; if (HAL_RCC_OscConfig(&RCC_OscInitStruct) != HAL_OK) { Error_Handler(); } /** Activate the Over-Drive mode */ if (HAL_PWREx_EnableOverDrive() != HAL_OK) { Error_Handler(); } /** Initializes the CPU, AHB and APB buses clocks */ RCC_ClkInitStruct.ClockType = RCC_CLOCKTYPE_HCLK¦RCC_CLOCKTYPE_SYSCLK ¦RCC_CLOCKTYPE_PCLK1¦RCC_CLOCKTYPE_PCLK2; RCC_ClkInitStruct.SYSCLKSource = RCC_SYSCLKSOURCE_PLLCLK; RCC_ClkInitStruct.AHBCLKDivider = RCC_SYSCLK_DIV1; RCC_ClkInitStruct.APB1CLKDivider = RCC_HCLK_DIV2; RCC_ClkInitStruct.APB2CLKDivider = RCC_HCLK_DIV2; if (HAL_RCC_ClockConfig(&RCC_ClkInitStruct, FLASH_LATENCY_2) != HAL_OK) { Error_Handler(); } } /* USER CODE BEGIN 4 */ void process_command_message(uint8_t command) { if (command == '1' ¦¦ command == '2') { sprintf(output_buffer, "Mode: %s\n", mode_name[current_mode]); } else if (command == '3' ¦¦ command == '4') { sprintf(output_buffer, "Speed: %ld%%\n", speed_percents); } else if (command == '5') { sprintf(output_buffer, "Mode: %s\nSpeed: %ld%%\n", mode_name[current_mode], speed_percents); } else if (command == '\r') { sprintf(output_buffer, "Custom menu is open\nEnter sequence: <color> <brightness> <duration>\n"); } else if (command == 0x1B) { sprintf(output_buffer, "Main menu is open\n"); } HAL_UART_Transmit(&huart6, output_buffer, strlen(output_buffer), 100); } void process_main_menu_command(uint8_t command) { if (command == '1') { if (current_mode == CUSTOM_MODE) { current_mode = BREATH; } else { current_mode++; } } else if (command == '2') { if (current_mode == BREATH) { current_mode = CUSTOM_MODE; } else { current_mode--; } } else if (command == '3') { if (speed_percents <= 90) { speed_percents += 10; } } else if (command == '4') { if (speed_percents >= 10) { speed_percents -= 10; } } else if (command == '\r') { current_menu_state = CUSTOM; } process_command_message(command); } void process_custom_menu_input(uint8_t symbol) { static char custom_menu_buffer[256]; static uint8_t custom_menu_index = 0; static _Bool is_first_input = 1; // Обработка ESC - выход из режима настройки if (symbol == 0x1B) { // ESC current_menu_state = MAIN; is_first_input = 1; custom_menu_index = 0; memset(custom_menu_buffer, 0, sizeof(custom_menu_buffer)); process_command_message(symbol); return; } // Обработка Backspaces if (symbol == 0x08 ¦¦ symbol == 0x7F) { // Backspace или Delete if (custom_menu_index > 0) { custom_menu_index--; custom_menu_buffer[custom_menu_index] = 0; // Отправляем последовательность для удаления символа в терминале char bs[] = "\b \b"; HAL_UART_Transmit(&huart6, bs, strlen(bs), 100); } return; } // Обработка Enter - парсинг введенной строки if (symbol == '\r' ¦¦ symbol == '\n') { if (custom_menu_index == 0) { sprintf(output_buffer, "\r\nNo input! Press ESC to cancel or enter sequence: \n"); HAL_UART_Transmit(&huart6, output_buffer, strlen(output_buffer), 100); return; } // Парсим введенную строку char color_char; uint32_t brightness; uint32_t duration; int parsed = sscanf(custom_menu_buffer, "%c %lu %lu", &color_char, &brightness, &duration); if (parsed != 3) { sprintf(output_buffer, "\r\nInvalid format! Use: <color> <brightness> <duration>\r\n"); HAL_UART_Transmit(&huart6, (uint8_t*)output_buffer, strlen(output_buffer), 100); custom_menu_index = 0; memset(custom_menu_buffer, 0, sizeof(custom_menu_buffer)); return; } // Валидация цвета indicator_color color; if (color_char == 'r' ¦¦ color_char == 'R') { color = RED; } else if (color_char == 'y' ¦¦ color_char == 'Y') { color = YELLOW; } else if (color_char == 'g' ¦¦ color_char == 'G') { color = GREEN; } else { sprintf(output_buffer, "\r\nInvalid color '%c'! Use r, y or g\r\n", color_char); HAL_UART_Transmit(&huart6, output_buffer, strlen(output_buffer), 100); custom_menu_index = 0; memset(custom_menu_buffer, 0, sizeof(custom_menu_buffer)); return; } // Валидация яркости if (brightness < 0 ¦¦ brightness > 100) { sprintf(output_buffer, "\nInvalid brightness %lu! Use 0-100\n", brightness); HAL_UART_Transmit(&huart6, output_buffer, strlen(output_buffer), 100); custom_menu_index = 0; memset(custom_menu_buffer, 0, sizeof(custom_menu_buffer)); return; } if (duration <= 0) { sprintf(output_buffer, "\nInvalid duration %lu! Must be > 0\n", duration); HAL_UART_Transmit(&huart6, output_buffer, strlen(output_buffer), 100); custom_menu_index = 0; memset(custom_menu_buffer, 0, sizeof(custom_menu_buffer)); return; } if (is_first_input) { current_custom_mode_step_count = 0; is_first_input = 0; } if (current_custom_mode_step_count < MAX_CUSTOM_MODE_STEP_COUNT) { custom_mode_steps[current_custom_mode_step_count].color = color; custom_mode_steps[current_custom_mode_step_count].brightness = brightness; custom_mode_steps[current_custom_mode_step_count].duration = duration; sprintf(output_buffer, "\nStep %d added: %s, %lu%%, %lums\n", current_custom_mode_step_count + 1, (color == RED) ? "RED" : (color == YELLOW) ? "YELLOW" : "GREEN", brightness, duration); HAL_UART_Transmit(&huart6, output_buffer, strlen(output_buffer), 100); current_custom_mode_step_count++; if (current_custom_mode_step_count == MAX_CUSTOM_MODE_STEP_COUNT) { sprintf(output_buffer, "Maximum steps reached (%d). Press ESC to finish.\n", MAX_CUSTOM_MODE_STEP_COUNT); HAL_UART_Transmit(&huart6, output_buffer, strlen(output_buffer), 100); current_menu_state = MAIN; is_first_input = 1; } else { sprintf(output_buffer, "Enter next step or press ESC to finish:\n"); HAL_UART_Transmit(&huart6, output_buffer, strlen(output_buffer), 100); } } else { sprintf(output_buffer, "Maximum steps reached (%d). Press ESC to finish.\n", MAX_CUSTOM_MODE_STEP_COUNT); HAL_UART_Transmit(&huart6, output_buffer, strlen(output_buffer), 100); current_menu_state = MAIN; is_first_input = 1; } custom_menu_index = 0; memset(custom_menu_buffer, 0, sizeof(custom_menu_buffer)); return; } if (custom_menu_index < sizeof(custom_menu_buffer) - 1 && symbol >= 32 && symbol <= 126) { custom_menu_buffer[custom_menu_index++] = symbol; HAL_UART_Transmit(&huart6, &symbol, 1, 100); } } void HAL_UART_RxCpltCallback(UART_HandleTypeDef *huart) { if (huart == &huart6) { if (current_menu_state == MAIN) { process_main_menu_command(input_buffer); } else { process_custom_menu_input(input_buffer); } HAL_UART_Receive_IT(&huart6, &input_buffer, 1); } } void wait(uint32_t delay) { if (delay == 0) return; uint32_t start_tick = HAL_GetTick(); while (HAL_GetTick() - start_tick < delay) {} } void play_breath() { static uint32_t brightness = 0; static uint32_t max_delay = 110; static _Bool going_up = 1; if (speed_percents == 0) { return; } htim4.Instance->CCR2 = brightness; htim4.Instance->CCR3 = 0; htim4.Instance->CCR4 = 0; uint32_t actual_delay = max_delay * (110 - speed_percents) / 100; wait(actual_delay); if (going_up) { brightness += 10; if (brightness >= MAX_BRIGHTNESS) { brightness = MAX_BRIGHTNESS; going_up = 0; } } else { brightness -= 10; if (brightness <= MIN_BRIGHTNESS) { brightness = MIN_BRIGHTNESS; going_up = 1; } } } void play_strobe() { static uint32_t max_delay = 1000; if (speed_percents == 0) { return; } htim4.Instance->CCR3 = 0; htim4.Instance->CCR4 = 0; htim4.Instance->CCR2 = 100; // �?справлено: инвертированная формула скорости uint32_t actual_delay = max_delay * (110 - speed_percents) / 100; wait(actual_delay); htim4.Instance->CCR2 = 0; wait(actual_delay); } void play_siren() { static uint32_t brightness = 0; static uint32_t max_delay = 110; static _Bool going_up = 1; if (speed_percents == 0) { return; } htim4.Instance->CCR2 = brightness; htim4.Instance->CCR3 = 100 - brightness; htim4.Instance->CCR4 = 0; // �?справлено: инвертированная формула скорости uint32_t actual_delay = max_delay * (110 - speed_percents) / 100; wait(actual_delay); if (going_up) { brightness += 10; if (brightness >= MAX_BRIGHTNESS) { brightness = MAX_BRIGHTNESS; going_up = 0; } } else { brightness -= 10; if (brightness <= MIN_BRIGHTNESS) { brightness = MIN_BRIGHTNESS; going_up = 1; } } } void play_every_signal() { static uint16_t pin = GPIO_PIN_13; static uint32_t max_delay = 1000; if (speed_percents == 0) { return; } if (pin == GPIO_PIN_13) { htim4.Instance->CCR2 = 100; htim4.Instance->CCR3 = 0; htim4.Instance->CCR4 = 0; pin = GPIO_PIN_14; } else if (pin == GPIO_PIN_14) { htim4.Instance->CCR2 = 0; htim4.Instance->CCR3 = 100; htim4.Instance->CCR4 = 0; pin = GPIO_PIN_15; } else { htim4.Instance->CCR2 = 0; htim4.Instance->CCR3 = 0; htim4.Instance->CCR4 = 100; pin = GPIO_PIN_13; } // �?справлено: инвертированная формула скорости uint32_t actual_delay = max_delay * (110 - speed_percents) / 100; wait(actual_delay); } void play_custom_mode() { if (current_menu_state == CUSTOM) { return; } if (current_custom_mode_step_count == 0) { return; } custom_mode_step current_custom_mode_step = custom_mode_steps[current_custom_mode_step_index]; if (current_custom_mode_step.color == RED) { htim4.Instance->CCR2 = 0; htim4.Instance->CCR3 = 0; htim4.Instance->CCR4 = current_custom_mode_step.brightness; } else if (current_custom_mode_step.color == YELLOW) { htim4.Instance->CCR2 = 0; htim4.Instance->CCR3 = current_custom_mode_step.brightness; htim4.Instance->CCR4 = 0; } else { htim4.Instance->CCR2 = current_custom_mode_step.brightness; htim4.Instance->CCR3 = 0; htim4.Instance->CCR4 = 0; } wait(current_custom_mode_step.duration); current_custom_mode_step_index++; if (current_custom_mode_step_index >= current_custom_mode_step_count) { current_custom_mode_step_index = 0; } } /* USER CODE END 4 */ /** * @brief This function is executed in case of error occurrence. * @retval None */ void Error_Handler(void) { /* USER CODE BEGIN Error_Handler_Debug */ /* User can add his own implementation to report the HAL error return state */ __disable_irq(); while (1) { } /* USER CODE END Error_Handler_Debug */ } #ifdef USE_FULL_ASSERT /** * @brief Reports the name of the source file and the source line number * where the assert_param error has occurred. * @param file: pointer to the source file name * @param line: assert_param error line source number * @retval None */ void assert_failed(uint8_t *file, uint32_t line) { /* USER CODE BEGIN 6 */ /* User can add his own implementation to report the file name and line number, ex: printf("Wrong parameters value: file %s on line %d\r\n", file, line) */ /* USER CODE END 6 */ } #endif /* USE_FULL_ASSERT */

ЗАКЛЮЧЕНИЕ

В ходе работы удалось настроить и запрограммировать микроконтроллер STM32F427VIT6 для работы с таймерами.