Министерство науки и высшего образования Российской Федерации ФЕДЕРАЛЬНОЕ ГОСУДАРСТВЕННОЕ АВТОНОМНОЕ ОБРАЗОВАТЕЛЬНОЕ УЧРЕЖДЕНИЕ ВЫСШЕГО ОБРАЗОВАНИЯ НАЦИОНАЛЬНЫЙ ИССЛЕДОВАТЕЛЬСКИЙ УНИВЕРСИТЕТ ИТМО (НИУ ИТМО) Факультет программной инженерии и компьютерной техники

ОТЧЁТ О ЛАБОРАТОРНОЙ РАБОТЕ № 4 Проектирование вычислительных систем по теме: Интерфейс I 2C и матричная клавиатура Вариант 3

Выполнили: студенты группы P3434 Р. Д. Баянов Д. А. Кузнецов Проверил: преподаватель В. Ю. Пинкевич

Санкт-Петербург 2025

СОДЕРЖАНИЕ

Цель работы	3

Задание	4

Вариант	5

1 Описание организации программы и структуры драйверов, блок-схема прикладного алгоритма	6

2 Исходный код	8

ЗАКЛЮЧЕНИЕ	21

Цель работы

1. Получить базовые знания об интерфейсе I2C и особенностях передачи данных по данному интерфейсу.

2. Получить базовые знания об устройстве и принципах работы контроллера интерфейса I2C в микроконтроллерах и получить навыки его программирования.

Задание

Разработать программу, которая использует интерфейс I2C для считывания нажатий кнопок клавиатуры стенда SDK-1.1.

Подсистема опроса клавиатуры должна удовлетворять следующим требованиям:

– реализуется защита от дребезга;

– нажатие кнопки фиксируется сразу после того, как было обнаружено, что кнопка

нажата (с учетом защиты от дребезга), а не в момент отпускания кнопки; если

необходимо, долгое нажатие может фиксироваться отдельно;

– кнопка, которая удерживается дольше, чем один цикл опроса, не считается повторно

нажатой до тех пор, пока не будет отпущена (нет переповторов);

– распознается и корректно обрабатывается множественное нажатие (при нажатии более чем одной кнопки считается, что ни одна кнопка не нажата, если это не противоречит требованиям к программе);

– всем кнопкам назначаются коды от 1 до 12 (порядок на усмотрение исполнителей).

Программа должна иметь два режима работы, переключение между которыми производится по нажатию кнопки на боковой панели стенда:

– режим тестирования клавиатуры;

– прикладной режим.

Уведомление о смене режима выводится в UART.

В режиме тестирования клавиатуры программа выводит в UART коды нажатых кнопок.

В прикладном режиме программа обрабатывает нажатия кнопок и выполняет действия в соответствии с вариантом задания.

Вариант

Задания аналогичны вариантам лабораторной работы №3, за исключением того, что ввод символов должен выполняться не с клавиатуры через UART, а с помощью клавиатуры стенда. Выбор кнопок клавиатуры стенда, играющих роль кнопок клавиатуры компьютера должен выполняться по усмотрению исполнителей. В отчете необходимо привести описание функций кнопок в реализованной программе.

Описание организации программы и структуры драйверов, блок-схема прикладного алгоритма

Конфигурация I2C

Блок-схема

Исходный код

/* USER CODE BEGIN Header */ /** ****************************************************************************** * @file : main.c * @brief : Main program body ****************************************************************************** * @attention * * Copyright (c) 2025 STMicroelectronics. * All rights reserved. * * This software is licensed under terms that can be found in the LICENSE file * in the root directory of this software component. * If no LICENSE file comes with this software, it is provided AS-IS. * ****************************************************************************** */ /* USER CODE END Header */ /* Includes ------------------------------------------------------------------*/ #include "main.h" #include "i2c.h" #include "tim.h" #include "usart.h" #include "gpio.h" /* Private includes ----------------------------------------------------------*/ /* USER CODE BEGIN Includes */ /* USER CODE END Includes */ /* Private typedef -----------------------------------------------------------*/ /* USER CODE BEGIN PTD */ typedef enum { BREATH = 0, STROBE, SIREN, EVERY_SIGNAL, CUSTOM_MODE } mode; typedef enum { GREEN, YELLOW, RED } indicator_color; typedef struct { indicator_color color; uint32_t brightness; uint32_t duration; } custom_mode_step; typedef enum { MAIN, CUSTOM } menu_state; typedef enum { COLOR, BRIGHTNESS, DURATION } custom_menu_state; typedef enum { IN_PROGRESS, NOT_PRESSED, PRESSED, } button_state; typedef enum { TEST, NORMAL } keyboard_state; /* USER CODE END PTD */ /* Private define ------------------------------------------------------------*/ /* USER CODE BEGIN PD */ #define MIN_BRIGHTNESS 0 #define MAX_BRIGHTNESS 100 #define MAX_CUSTOM_MODE_STEP_COUNT 5 #define KB_I2C_ADDRESS (0xE2) #define KB_I2C_READ_ADDRESS ((KB_I2C_ADDRESS) ¦ 1) #define KB_I2C_WRITE_ADDRESS ((KB_I2C_ADDRESS) & ~1) #define KB_INPUT_REG (0x0) #define KB_OUTPUT_REG (0x1) #define KB_CONFIG_REG (0x3) #define KB_KEY_DEBOUNCE_TIME (200) #define ANTI_BOUNCE_DELAY 40 #define PRESS_DELAY 100 /* USER CODE END PD */ /* Private macro -------------------------------------------------------------*/ /* USER CODE BEGIN PM */ /* USER CODE END PM */ /* Private variables ---------------------------------------------------------*/ /* USER CODE BEGIN PV */ uint32_t speed_percents = 50; mode current_mode = BREATH; char* mode_name[] = { "breath", "strobe", "siren", "every signal", "custom mode" }; custom_mode_step custom_mode_steps[MAX_CUSTOM_MODE_STEP_COUNT]; uint32_t current_custom_mode_step_count = 0; uint32_t current_custom_mode_step_index = 0; uint8_t input_buffer; uint8_t output_buffer[256]; menu_state current_menu_state = MAIN; custom_menu_state current_custom_menu_state = COLOR; keyboard_state current_keyboard_state = NORMAL; uint32_t last_pressing_time = 0; int last_pressed_btn_index = -1; int stroka[4] = {0, 0, 0, 0}; int stolbec[3] = {0, 0, 0}; /* USER CODE END PV */ /* Private function prototypes -----------------------------------------------*/ void SystemClock_Config(void); /* USER CODE BEGIN PFP */ void wait(uint32_t delay); void play_breath(); void play_strobe(); void play_siren(); void play_every_signal(); void play_custom_mode(); void process_main_menu_command(int); void process_custom_menu_command(int); void process_keyboard_state_change(); void process_test_keyboard(int btn_index); int get_pressed_btn_index(); button_state check_button(); /* USER CODE END PFP */ /* Private user code ---------------------------------------------------------*/ /* USER CODE BEGIN 0 */ /* USER CODE END 0 */ /** * @brief The application entry point. * @retval int */ int main(void) { /* USER CODE BEGIN 1 */ /* USER CODE END 1 */ /* MCU Configuration--------------------------------------------------------*/ /* Reset of all peripherals, Initializes the Flash interface and the Systick. */ HAL_Init(); /* USER CODE BEGIN Init */ /* USER CODE END Init */ /* Configure the system clock */ SystemClock_Config(); /* USER CODE BEGIN SysInit */ /* USER CODE END SysInit */ /* Initialize all configured peripherals */ MX_GPIO_Init(); MX_I2C1_Init(); MX_USART6_UART_Init(); MX_TIM4_Init(); /* USER CODE BEGIN 2 */ /* USER CODE END 2 */ /* Infinite loop */ /* USER CODE BEGIN WHILE */ HAL_TIM_PWM_Start(&htim4, TIM_CHANNEL_2); HAL_TIM_PWM_Start(&htim4, TIM_CHANNEL_3); HAL_TIM_PWM_Start(&htim4, TIM_CHANNEL_4); HAL_UART_Receive_IT(&huart6, &input_buffer, 1); while (1) { if (check_button() == PRESSED) { process_keyboard_state_change(); } int btn_index = get_pressed_btn_index(); if (btn_index != -1) {\ if (current_keyboard_state == TEST) { process_test_keyboard(btn_index); } else { if (current_menu_state == MAIN) { process_main_menu_command(btn_index); } else { process_custom_menu_input(btn_index); } } } if (current_mode == BREATH) { play_breath(); } else if (current_mode == STROBE) { play_strobe(); } else if (current_mode == SIREN) { play_siren(); } else if (current_mode == EVERY_SIGNAL) { play_every_signal(); } else { play_custom_mode(); } /* USER CODE END WHILE */ /* USER CODE BEGIN 3 */ } /* USER CODE END 3 */ } /** * @brief System Clock Configuration * @retval None */ void SystemClock_Config(void) { RCC_OscInitTypeDef RCC_OscInitStruct = {0}; RCC_ClkInitTypeDef RCC_ClkInitStruct = {0}; /** Configure the main internal regulator output voltage */ __HAL_RCC_PWR_CLK_ENABLE(); __HAL_PWR_VOLTAGESCALING_CONFIG(PWR_REGULATOR_VOLTAGE_SCALE1); /** Initializes the RCC Oscillators according to the specified parameters * in the RCC_OscInitTypeDef structure. */ RCC_OscInitStruct.OscillatorType = RCC_OSCILLATORTYPE_HSE; RCC_OscInitStruct.HSEState = RCC_HSE_ON; RCC_OscInitStruct.PLL.PLLState = RCC_PLL_ON; RCC_OscInitStruct.PLL.PLLSource = RCC_PLLSOURCE_HSE; RCC_OscInitStruct.PLL.PLLM = 15; RCC_OscInitStruct.PLL.PLLN = 216; RCC_OscInitStruct.PLL.PLLP = RCC_PLLP_DIV2; RCC_OscInitStruct.PLL.PLLQ = 4; if (HAL_RCC_OscConfig(&RCC_OscInitStruct) != HAL_OK) { Error_Handler(); } /** Activate the Over-Drive mode */ if (HAL_PWREx_EnableOverDrive() != HAL_OK) { Error_Handler(); } /** Initializes the CPU, AHB and APB buses clocks */ RCC_ClkInitStruct.ClockType = RCC_CLOCKTYPE_HCLK¦RCC_CLOCKTYPE_SYSCLK ¦RCC_CLOCKTYPE_PCLK1¦RCC_CLOCKTYPE_PCLK2; RCC_ClkInitStruct.SYSCLKSource = RCC_SYSCLKSOURCE_PLLCLK; RCC_ClkInitStruct.AHBCLKDivider = RCC_SYSCLK_DIV1; RCC_ClkInitStruct.APB1CLKDivider = RCC_HCLK_DIV4; RCC_ClkInitStruct.APB2CLKDivider = RCC_HCLK_DIV2; if (HAL_RCC_ClockConfig(&RCC_ClkInitStruct, FLASH_LATENCY_5) != HAL_OK) { Error_Handler(); } } /* USER CODE BEGIN 4 */ void process_command_message(int command) { if (command == 1 ¦¦ command == 2) { sprintf(output_buffer, "\r\nMode: %s\n", mode_name[current_mode]); } else if (command == 3 ¦¦ command == 4) { sprintf(output_buffer, "\r\nSpeed: %ld%%\n", speed_percents); } else if (command == 5) { sprintf(output_buffer, "\r\nMode: %s\nSpeed: %ld%%\n", mode_name[current_mode], speed_percents); } else if (command == 12) { sprintf(output_buffer, "\r\nCustom menu mode is open\nChosen step: RED, 50%, 100ms\n"); } if (command == 1 ¦¦ command == 2 ¦¦ command == 3 ¦¦ command == 4 ¦¦ command == 5 ¦¦ command == 12) { HAL_UART_Transmit(&huart6, output_buffer, strlen(output_buffer), 100); } } void process_main_menu_command(int command) { if (command == 1) { if (current_mode == CUSTOM_MODE) { current_mode = BREATH; } else { current_mode++; } } else if (command == 2) { if (current_mode == BREATH) { current_mode = CUSTOM_MODE; } else { current_mode--; } } else if (command == 3) { if (speed_percents <= 90) { speed_percents += 10; } } else if (command == 4) { if (speed_percents >= 10) { speed_percents -= 10; } } else if (command == 12) { current_menu_state = CUSTOM; } process_command_message(command); } void process_custom_menu_input(int command) { static _Bool is_first_input = 1; static indicator_color chosen_color = RED; static uint32_t chosen_brightness = 50; static uint32_t chosen_duration = 100; // Обработка ESC - выход из режима настройки if (command == 12) { // ESC current_menu_state = MAIN; is_first_input = 1; chosen_color = RED; chosen_brightness = 50; chosen_duration = 100; sprintf(output_buffer, "Main menu is open\n"); HAL_UART_Transmit(&huart6, output_buffer, strlen(output_buffer), 100); return; } sprintf(output_buffer, "Chosen step: %s, %lu%%, %lums\n", (chosen_color == RED) ? "RED" : (chosen_color == YELLOW) ? "YELLOW" : "GREEN", chosen_brightness, chosen_duration); HAL_UART_Transmit(&huart6, output_buffer, strlen(output_buffer), 100); // Обработка Enter - парсинг введенной строки if (command == 3) { if (current_custom_menu_state == COLOR) { current_custom_menu_state = BRIGHTNESS; } else if (current_custom_menu_state == BRIGHTNESS) { current_custom_menu_state = DURATION; } else { current_custom_menu_state = COLOR; if (is_first_input) { current_custom_mode_step_count = 0; is_first_input = 0; } if (current_custom_mode_step_count < MAX_CUSTOM_MODE_STEP_COUNT) { custom_mode_steps[current_custom_mode_step_count].color = chosen_color; custom_mode_steps[current_custom_mode_step_count].brightness = chosen_brightness; custom_mode_steps[current_custom_mode_step_count].duration = chosen_duration; sprintf(output_buffer, "\nStep %d added: %s, %lu%%, %lums\n", current_custom_mode_step_count + 1, (chosen_color == RED) ? "RED" : (chosen_color == YELLOW) ? "YELLOW" : "GREEN", chosen_brightness, chosen_duration); HAL_UART_Transmit(&huart6, output_buffer, strlen(output_buffer), 100); current_custom_mode_step_count++; if (current_custom_mode_step_count == MAX_CUSTOM_MODE_STEP_COUNT) { sprintf(output_buffer, "\r\nMaximum steps reached (%d). Press ESC to finish.\n", MAX_CUSTOM_MODE_STEP_COUNT); HAL_UART_Transmit(&huart6, output_buffer, strlen(output_buffer), 100); current_menu_state = MAIN; is_first_input = 1; } else { sprintf(output_buffer, "\r\nEnter next step or press ESC to finish:\n"); HAL_UART_Transmit(&huart6, output_buffer, strlen(output_buffer), 100); } } else { sprintf(output_buffer, "\r\nMaximum steps reached (%d). Press ESC to finish.\n", MAX_CUSTOM_MODE_STEP_COUNT); HAL_UART_Transmit(&huart6, output_buffer, strlen(output_buffer), 100); current_menu_state = MAIN; is_first_input = 1; } chosen_color = RED; chosen_brightness = 50; chosen_duration = 100; } } if (command == 1) { if (current_custom_menu_state == COLOR) { if (chosen_color == RED) { chosen_color = GREEN; } else if (chosen_color == GREEN) { chosen_color = YELLOW; } else { chosen_color = RED; } sprintf(output_buffer, "Chosen color: %s\n", (chosen_color == RED) ? "RED" : (chosen_color == YELLOW) ? "YELLOW" : "GREEN"); } else if (current_custom_menu_state == BRIGHTNESS) { if (chosen_brightness != 0) { chosen_brightness -= 10; } sprintf(output_buffer, "Chosen brightness: %d\n", chosen_brightness); } else { if (chosen_duration != 0) { chosen_duration -= 100; } sprintf(output_buffer, "Chosen duration: %d\n", chosen_duration); } HAL_UART_Transmit(&huart6, output_buffer, strlen(output_buffer), 100); } if (command == 2) { if (current_custom_menu_state == COLOR) { if (chosen_color == RED) { chosen_color = YELLOW; } else if (chosen_color == GREEN) { chosen_color = RED; } else { chosen_color = GREEN; } sprintf(output_buffer, "Chosen color: %s\n", (chosen_color == RED) ? "RED" : (chosen_color == YELLOW) ? "YELLOW" : "GREEN"); } else if (current_custom_menu_state == BRIGHTNESS) { if (chosen_brightness != 100) { chosen_brightness += 10; } sprintf(output_buffer, "Chosen brightness: %d\n", chosen_brightness); } else { if (chosen_duration != 1000) { chosen_duration += 100; } sprintf(output_buffer, "Chosen duration: %d\n", chosen_duration); } HAL_UART_Transmit(&huart6, output_buffer, strlen(output_buffer), 100); } } void process_keyboard_state_change() { if (current_keyboard_state == NORMAL) { current_keyboard_state = TEST; sprintf(output_buffer, "Keyboard test mode activated\n"); } else { current_keyboard_state = NORMAL; sprintf(output_buffer, "Keyboard normal mode activated\n"); } HAL_UART_Transmit(&huart6, output_buffer, strlen(output_buffer), 100); } void process_test_keyboard(int btn_index) { sprintf(output_buffer, "Pressed button %d\n", btn_index); HAL_UART_Transmit(&huart6, output_buffer, strlen(output_buffer), 100); } void HAL_UART_RxCpltCallback(UART_HandleTypeDef *huart) { if (huart == &huart6) { if (current_menu_state == MAIN) { process_main_menu_command(input_buffer); } else { process_custom_menu_input(input_buffer); } HAL_UART_Receive_IT(&huart6, &input_buffer, 1); } } void wait(uint32_t delay) { if (delay == 0) return; uint32_t start_tick = HAL_GetTick(); while (HAL_GetTick() - start_tick < delay) { int btn_index = get_pressed_btn_index(); if (btn_index != -1) { process_main_menu_command(btn_index); return; } } } button_state check_button() { static _Bool was_pressed = 0; static uint32_t start_time = 0; if (HAL_GPIO_ReadPin(GPIOC, GPIO_PIN_15) == GPIO_PIN_RESET) { /* Кнопка на текущий момент нажата */ if (!was_pressed) { was_pressed = 1; start_time = HAL_GetTick(); } return IN_PROGRESS; } else { /* Кнопка на текущий момент не нажата */ if (!was_pressed) { return NOT_PRESSED; } was_pressed = 0; uint32_t current_delay = HAL_GetTick() - start_time; if (current_delay >= PRESS_DELAY) { return PRESSED; } } } void play_breath() { static uint32_t brightness = 0; static uint32_t max_delay = 110; static _Bool going_up = 1; if (speed_percents == 0) { return; } htim4.Instance->CCR2 = brightness; htim4.Instance->CCR3 = 0; htim4.Instance->CCR4 = 0; uint32_t actual_delay = max_delay * (110 - speed_percents) / 100; wait(actual_delay); if (going_up) { brightness += 10; if (brightness >= MAX_BRIGHTNESS) { brightness = MAX_BRIGHTNESS; going_up = 0; } } else { brightness -= 10; if (brightness <= MIN_BRIGHTNESS) { brightness = MIN_BRIGHTNESS; going_up = 1; } } } void play_strobe() { static uint32_t max_delay = 1000; if (speed_percents == 0) { return; } htim4.Instance->CCR3 = 0; htim4.Instance->CCR4 = 0; htim4.Instance->CCR2 = 100; // �?справлено: инвертированная формула скорости uint32_t actual_delay = max_delay * (110 - speed_percents) / 100; wait(actual_delay); htim4.Instance->CCR2 = 0; wait(actual_delay); } void play_siren() { static uint32_t brightness = 0; static uint32_t max_delay = 110; static _Bool going_up = 1; if (speed_percents == 0) { return; } htim4.Instance->CCR2 = brightness; htim4.Instance->CCR3 = 100 - brightness; htim4.Instance->CCR4 = 0; // �?справлено: инвертированная формула скорости uint32_t actual_delay = max_delay * (110 - speed_percents) / 100; wait(actual_delay); if (going_up) { brightness += 10; if (brightness >= MAX_BRIGHTNESS) { brightness = MAX_BRIGHTNESS; going_up = 0; } } else { brightness -= 10; if (brightness <= MIN_BRIGHTNESS) { brightness = MIN_BRIGHTNESS; going_up = 1; } } } void play_every_signal() { static uint16_t pin = GPIO_PIN_13; static uint32_t max_delay = 1000; if (speed_percents == 0) { return; } if (pin == GPIO_PIN_13) { htim4.Instance->CCR2 = 100; htim4.Instance->CCR3 = 0; htim4.Instance->CCR4 = 0; pin = GPIO_PIN_14; } else if (pin == GPIO_PIN_14) { htim4.Instance->CCR2 = 0; htim4.Instance->CCR3 = 100; htim4.Instance->CCR4 = 0; pin = GPIO_PIN_15; } else { htim4.Instance->CCR2 = 0; htim4.Instance->CCR3 = 0; htim4.Instance->CCR4 = 100; pin = GPIO_PIN_13; } // �?справлено: инвертированная формула скорости uint32_t actual_delay = max_delay * (110 - speed_percents) / 100; wait(actual_delay); } void play_custom_mode() { if (current_menu_state == CUSTOM) { return; } if (current_custom_mode_step_count == 0) { return; } custom_mode_step current_custom_mode_step = custom_mode_steps[current_custom_mode_step_index]; if (current_custom_mode_step.color == RED) { htim4.Instance->CCR2 = 0; htim4.Instance->CCR3 = 0; htim4.Instance->CCR4 = current_custom_mode_step.brightness; } else if (current_custom_mode_step.color == YELLOW) { htim4.Instance->CCR2 = 0; htim4.Instance->CCR3 = current_custom_mode_step.brightness; htim4.Instance->CCR4 = 0; } else { htim4.Instance->CCR2 = current_custom_mode_step.brightness; htim4.Instance->CCR3 = 0; htim4.Instance->CCR4 = 0; } wait(current_custom_mode_step.duration); current_custom_mode_step_index++; if (current_custom_mode_step_index >= current_custom_mode_step_count) { current_custom_mode_step_index = 0; } } int get_pressed_btn_index(){ const uint32_t t = HAL_GetTick(); if (t - last_pressing_time < KB_KEY_DEBOUNCE_TIME) return -1; int index = -1; uint8_t reg_buffer = ~0; uint8_t tmp = 0; int i = 0; HAL_I2C_Mem_Write(&hi2c1, KB_I2C_WRITE_ADDRESS, KB_OUTPUT_REG, 1, &tmp, 1, KB_KEY_DEBOUNCE_TIME); for (int row = 0; row < 4; row++) { uint8_t buf = ~((uint8_t) (1 << row)); HAL_I2C_Mem_Write(&hi2c1, KB_I2C_WRITE_ADDRESS, KB_CONFIG_REG, 1, &buf, 1, KB_KEY_DEBOUNCE_TIME); HAL_Delay(10); HAL_I2C_Mem_Read(&hi2c1, KB_I2C_READ_ADDRESS, KB_INPUT_REG, 1, &reg_buffer, 1, KB_KEY_DEBOUNCE_TIME); switch(reg_buffer >> 4){ case 6: index = (i == 0) ? row * 3 + 1 : -1; i++; stroka[row] += 1; stolbec[0] += 1; break; case 5: index = (i == 0) ? row * 3 + 2 : -1; i++; stroka[row] += 1; stolbec[1] += 1; break; case 3: index = (i == 0) ? row * 3 + 3 : -1; i++; stroka[row] += 1; stolbec[2] += 1; break; case 0: case 1: case 2: case 4: index = -1; } } if (index != -1) last_pressing_time = t; if (index == last_pressed_btn_index){ return -1; } last_pressed_btn_index = index; int sum_stroka = 0; int sum_stolbec = 0; for (int j=0; j <= 3; j++){ sum_stroka += stroka[j]; stroka[j] = 0; } for (int j=0; j <= 2; j++){ sum_stolbec += stolbec[j]; stolbec[j] = 0; } if (sum_stroka != 1){ return -1; } if (sum_stolbec != 1){ return -1; } return index; } /* USER CODE END 4 */ /** * @brief This function is executed in case of error occurrence. * @retval None */ void Error_Handler(void) { /* USER CODE BEGIN Error_Handler_Debug */ /* User can add his own implementation to report the HAL error return state */ __disable_irq(); while (1) { } /* USER CODE END Error_Handler_Debug */ } #ifdef USE_FULL_ASSERT /** * @brief Reports the name of the source file and the source line number * where the assert_param error has occurred. * @param file: pointer to the source file name * @param line: assert_param error line source number * @retval None */ void assert_failed(uint8_t *file, uint32_t line) { /* USER CODE BEGIN 6 */ /* User can add his own implementation to report the file name and line number, ex: printf("Wrong parameters value: file %s on line %d\r\n", file, line) */ /* USER CODE END 6 */ } #endif /* USE_FULL_ASSERT */

ЗАКЛЮЧЕНИЕ

В ходе работы удалось настроить и запрограммировать микроконтроллер STM32F427VIT6 для работы c интерфейсом I2C и матричной клавиатурой.